# Chemical Reaction Engineering - II

# Assignment 1

# 1. Objective

The objective is to find the kinetic model that provides the best statistical and physically reasonable description of the assigned dataset for CO2 to methanol process.

# 2. Datasets Allocation

Each student will work with only one of the two datasets, determined by the last digit of their entry number:

- Odd entry numbers: Dataset A

- Even entry numbers: Dataset B

Additional Information:

- Dataset A – Data obtained over a Cu/ZnO/Al2O3 catalyst in a plug-flow reactor with a catalyst bed length of 200 mm, catalyst weight – 1.5 g, tube diameter – 12 mm, and a concentric rod (thermocouple) of 2 mm diameter. Volume Flow is at 298 K, 1 bar.

- Dataset B – Data obtained over a Cu/ZnO/ZrO2 catalyst in a plug-flow reactor with a catalyst bed length of 26 mm, catalyst weight – 1 g, tube diameter – 17.4 mm, and a concentric rod (thermocouple) of 3.175 mm diameter. Volume Flow is at 273 K, 1 atm.

# 3. Task

A Langmuir Hinshelwood Hougen Watson (LHHW) kinetic model for methanol synthesis, along with an initial set of kinetic parameters, is provided below. The task is to predict the outlet mol fraction of CO, CO2, and methanol, and refine the kinetic model to best describe the assigned dataset. You are free to:

- re-estimate all the kinetic parameters,

- re-estimate only a subset of the parameters while holding the rest fixed at their provided values,

- adjust parameters selectively, guided by physical and statistical reasoning,

- modify the provided rate expression.

- consider only a few of the below-mentioned reactions or add new reactions

# 4. Information provided

Excel file: Data.xlsx contains DATASET A and B.

MATLAB codes: Code_A predicts the outlet molar fractions using DATASET A, and Code_B predicts the same using DATASET B, using the base set of kinetic parameters.

Reactions:

- CO2 hydrogenation:

CO2+3H2→CH3OH+H2O

- RWGS reaction:

CO2+H2→CO+H2O

- CO hydrogenation:

CO+2H2→CH3OH

Corresponding kinetic expressions are:

rCO2 hydrogenation=kCO2KCO2fCO2fH21.5 - 1Keq,CO2.fMeOHfH2OfH2-1.5DEN

rRWGS=kRWGSKCO2fCO2fH2  -  1Keq,RWGS.fH2OfCODEN

rCO hydrogenation=kCOKCOfCOfH21.5  -  1Keq,CO.fMeOHfH2-0.5DEN

DEN=(1+KCOfCO+KCO2fCO2)(fH20.5+KH2OKH20.5.fH2O)

Where,

k=koexp-EaRT

K=Koexp-∆HadsRT

Keq=Keexp-∆HrxnRT

Kinetic parameters:

| S. No. | Kinetic parameter | ko | Ea |
| --- | --- | --- | --- |
| 1 | kCO2 | 9.092×102 | 45889 |
| 2 | kRWGS | 7.898×1013 | 149856 |
| 3 | kCO | 4.845×106 | 106729 |
|  | Adsorption constants | Ko | ΔHads |
| 4 | KCO2 | 4.101×10-9 | -76594 |
| 5 | KCO | 8.641×10-3 | -14936 |
| 6 | KH2OKH2 | 2.333×10-8 | -97350 |
|  | Equilibrium constants | Ke | ΔHrxn |
| 7 | Keq,CO2 | 2.547×10-11 | -58705 |
| 8 | Keq,RWGS | 106.996 | 39692 |
| 9 | Keq,CO | 2.381×10-13 | -98397 |

# 5. Report Guidelines

The report should include:

- Methodology followed, including the algorithm for parameter estimation.

- Performance metrics used to evaluate the model (R², root mean square error (RMSE), mean absolute error (MAE), and parity plots of predicted vs. experimental outlet compositions).

- A table containing the comparison of the estimated values of all kinetic parameters and the originally provided values.

- Discussion of the results obtained, including justification of any changes made to the reaction scheme and/or the form of the rate expressions (if applicable).

- The report should strictly be limited to less than 3 pages.

# 6. Submission Guidelines

Submit the report in a zip file and name it according to your entry number, eg, 2021CHE8174.zip

Zip file should contain the following:

- A PDF of the report (less than 3 pages). Any content beyond 3 pages will not be evaluated.

- All scripts / code (original files) used for the parameter estimation. These files should be in native file formats (.m, .py, .cpp, .java)

Note:

1. Units of the parameters are provided in the Excel sheet.

2. Assume an isothermal and isobaric system.

3. Use of AI tools (e.g., ChatGPT, Copilot, Claude) to help write or debug code is permitted, provided you fully understand the logic, equations, and assumptions in the code you submit. You must be able to explain and justify this code as part of the Methodology description in your report.